Digital System Design Lab

Virtual Lab – 1

Aim: To design and implement a 2 bit Comparator using logic gates in Verilog.

Software Required: ModelSim

Procedure:

- Open ModelSim, Create new project, new file and type the name.

- Select Verilog in that drop-down menu.

- Type the code, compile it.

- Type the test bench for it and compile and simulate it.

- Click Add Wave and Create the waveform.

- Repeat this steps for different modelling.

Truth Table:

The truth table is given below.

| Inputs |  |  |  | X | Y | Z |
| --- | --- | --- | --- | --- | --- | --- |
| A1(MSB) | A0 | B1(MSB) | B0 | A<B | A>B | A=B |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 |

Two 2 bit numbers A1,A0 and B1,B0 are compared with each other. X, Y and Z are outputs.

X=1 when A<B and 0 otherwise; Y=1 when A>B and 0 otherwise; and Z=1 when A=B and 0 otherwise.

K- Map:

X = A1'A0'B0 + A0'B1B0 + A1'B1

Y = A0B1'B0' + A1A0B0' + A1B1'

Z = (A0 ⊙ B0)(A1 ⊙ B1)

Logic Diagram:

Verilog Code:

Gate Modelling:

module vlgm (A1,A0,B1,B0,X,Y,Z);

input A1,A0,B1,B0;

output X,Y,Z;

wire i1,i2,i3,i4,i5,i6,i7,i8,i9,i10,i11,i12;

not a1 (i1,A0);

not a2 (i2,A1);

not a3 (i3,B0);

not a4 (i4,B1);

and a5 (i5,i1,i2,B0);

and a6 (i6,i1,B0,B1);

and a7 (i7,i2,B1);

or a8 (X,i5,i6,i7);

and a9 (i8,A0,A1,i3);

and a10 (i9,A0,i3,i4);

and a11 (i10,A1,i4);

or a12 (Y,i8,i9,i10);

xnor a13 (i11,A0,B0);

xnor a14 (i12,A1,B1);

and a15 (Z,i11,i12);

endmodule

Data Flow Modelling:

module vldfm (A1,A0,B1,B0,X,Y,Z);

input A1,A0,B1,B0;

output X,Y,Z;

assign X = (~A0&~A1&B0)|(~A0&B0&B1)|(~A1&B1);

assign Y = (A0&A1&~B0)|(A0&~B0&~B1)|(A1&~B1);

assign Z = (A0~^B0)&(A1~^B1);

endmodule

Behavioural Modelling:

module vlbm (A1,A0,B1,B0,X,Y,Z);

input A1,A0,B1,B0;

output reg X,Y,Z;

always @ (A1,A0,B1,B0)

case ({A1,A0,B1,B0})

4'b0000: begin X = 0; Y = 0; Z = 1; end

4'b0001: begin X = 1; Y = 0; Z = 0; end

4'b0010: begin X = 1; Y = 0; Z = 0; end

4'b0011: begin X = 1; Y = 0; Z = 0; end

4'b0100: begin X = 0; Y = 1; Z = 0; end

4'b0101: begin X = 0; Y = 0; Z = 1; end

4'b0110: begin X = 1; Y = 0; Z = 0; end

4'b0111: begin X = 1; Y = 0; Z = 0; end

4'b1000: begin X = 0; Y = 1; Z = 0; end

4'b1001: begin X = 0; Y = 1; Z = 0; end

4'b1010: begin X = 0; Y = 0; Z = 1; end

4'b1011: begin X = 1; Y = 0; Z = 0; end

4'b1100: begin X = 0; Y = 1; Z = 0; end

4'b1101: begin X = 0; Y = 1; Z = 0; end

4'b1110: begin X = 0; Y = 1; Z = 0; end

4'b1111: begin X = 0; Y = 0; Z = 1; end

endcase

endmodule

Test Bench:

module vltb;

reg A1,A0,B1,B0;

wire X,Y,Z;

vlgm dut (.A1(A1),.A0(A0),.B1(B1),.B0(B0),.X(X),.Y(Y),.Z(Z));

initial begin

$monitor (A1,A0,B1,B0,X,Y,Z);

A1 = 1'b0;

A0 = 1'b0;

B1 = 1'b0;

B0 = 1'b0;

#100

A1 = 1'b0;

A0 = 1'b0;

B1 = 1'b0;

B0 = 1'b1;

#100

A1 = 1'b0;

A0 = 1'b0;

B1 = 1'b1;

B0 = 1'b0;

#100

A1 = 1'b0;

A0 = 1'b0;

B1 = 1'b1;

B0 = 1'b1;

#100

A1 = 1'b0;

A0 = 1'b1;

B1 = 1'b0;

B0 = 1'b0;

#100

A1 = 1'b0;

A0 = 1'b1;

B1 = 1'b0;

B0 = 1'b1;

#100

A1 = 1'b0;

A0 = 1'b1;

B1 = 1'b1;

B0 = 1'b0;

#100

A1 = 1'b0;

A0 = 1'b1;

B1 = 1'b1;

B0 = 1'b1;

#100

A1 = 1'b1;

A0 = 1'b0;

B1 = 1'b0;

B0 = 1'b0;

#100

A1 = 1'b1;

A0 = 1'b0;

B1 = 1'b0;

B0 = 1'b1;

#100

A1 = 1'b1;

A0 = 1'b0;

B1 = 1'b1;

B0 = 1'b0;

#100

A1 = 1'b1;

A0 = 1'b0;

B1 = 1'b1;

B0 = 1'b1;

#100

A1 = 1'b1;

A0 = 1'b1;

B1 = 1'b0;

B0 = 1'b0;

#100

A1 = 1'b1;

A0 = 1'b1;

B1 = 1'b0;

B0 = 1'b1;

#100A

A1 = 1'b1;

A0 = 1'b1;

B1 = 1'b0;

B0 = 1'b1;

#100

A1 = 1'b1;

A0 = 1'b1;

B1 = 1'b1;

B0 = 1'b1;

end

endmodule

Output:

Gate Modelling:

Data Flow Modelling:

Behavioural Modelling:

Result:

A 2 bit Comparator is designed and implemented using logic gates in Verilog.